- SECTION 22 67 19

- DriSteem®: Dechlorinator Systems

### PART 1 — GENERAL

#### 1.1	Summary

##### Section includes:

###### DriSteem dechlorinator which includes:

System controls

##### Related items:

###### Electrical Service and Connections: Division 26

#### 1.2	References

##### Quality assurance:

###### Electrical Components, Devices, and Accessories: Listed and labeled as defined in NFPA 70, Article 100, by a testing agency acceptable to authority having jurisdiction, and marked for intended use.

###### ISO 9001-2015 certified

#### 1.3	Submittals

##### Comply with Submittal Procedures and Execution and Closeout Requirements in General Requirements.

##### Submit product data (manufacturer’s specifications and technical data including performance, construction and fabrication) for each manufactured component.

#### 1.4	Warranty

##### Product shall be warranted to be free from defects in materials and fabrication for a period of two years after installation or 27 months from ship date.

### PART 2 — PRODUCTS

#### 2.1	System shall be DriSteem dechlorinator or engineer-approved equal and include all of the following integrated sub-systems:

System controls

##### Water pre-treatment: Water pre-treatment components and piping shall include:

###### Dechlorinators: Components, features, and functions shall include:

Granular, activated-carbon filter, 12 x 40 mesh, coconut based, compliant with ANSI B604

Electronic control valve(s) with integrated bypass valve (1” models)

One resin tank

Support gravel

One set of internal distribution

Drain line flow control based on the size of the resin tank

Solid-state microprocessor with front panel display

Micro-switch kit

##### Skid mounted (option): Water pretreatment selected with skid mounted option shall include the following:

###### Frame: Frame shall be steel, primed and finished with powder-coat epoxy paint. Frame shall be equipped for and support loads imposed during rigging and operation. Frame shall house resin tank, support piping, and electrical box.

###### System Components: System shall be placed on the frame and secured with straps.

###### Piping: The resin tank on the skid shall be plumbed at the manufacturer allowing for one connection, made in the field, for inlet, outlet, and drain. Pressure gauges, sampling valves, and bypass (if not integrated in the equipment) shall be included.

###### Electrical: Frame shall have a mounted electrical box for one point of connection to the power supply. The internal wiring shall be done by the manufacturer.

###### Support Legs: Frame shall come with thread able feet to allow for field leveling.

### PART 3 — EXECUTION

#### 3.1	Installation

##### Install per manufacturer's printed instructions and as indicated on drawings.

##### Coordinate electrical connections as specified in Division 26.

#### 3.2	Commissioning

##### The complete dechlorinator, as described above, shall be fully commissioned by factory-trained technicians. Commissioning of dechlorinator shall be coordinated with commissioning of the building automation system.

- END OF SECTION